# seen test cases by instLab 2: CS527

Release date: 7th August 2026

Submission date: 14th August 2026, 9AM.

# Problem Statement:

Simulator a mini-computer system.

Users can write a program in a simple programming language, as specified in the language specification section. A compiler will compile the program, and will generate a byte code. Byte code will be read by a computer system, and will initialise a memory. The processor will read the instruction one by one to execute the program, and will modify the memory. At the end, memory will be written to the computer system in a file.

Finally, compilation and computer system should be invoked by a single main file, and executed as a single executable.

All the changes with respect to Lab 1 is done in blue

Assume all the registers in the architecture are of 32 bits. Memory is byte addressable,

# Language specification:

Arithmetic operations

Language supports following basic math operations : + - / *

Dest Variable = <Operand 1 variable>   <operation> <Operand 2 variable>

Dest Variable = <Operand 1 variable>   <operation>  <constant value>

Example:

x1 =  x2 +  10

x1 =  x2 - 20

The maximum value of constant could be 255.

Every addition/subtraction operation updates the following four global flags: Z N C V

Z is 1, if result of operation is 0

N is 1, if MSB of result of operation is 1

C is 1, {For addition: if the unsigned result is strictly less than either of the unsigned inputs}. For subtraction: if operand 1 is bigger than operand 2.

V is 1, {for addition: if both operands sign is same, and sign of result is different than operand’s sign.} {For subtraction: if the operands have different signs, and the result's sign matches the second operand's original sign}

Memory operations: Read and Write

Read <variable>  <address>

Write <variable>  <address>   //address is a constant

Support of square brackets.

Dest Variable = [Operand 2 variable]    //memory read.

[Dest Variable address] = Operand 2 variable     //memory write to given address stored in a variable.

Operand could be a variable or a constant.

Example:

x1 = [x2]

[x2] = x1;

x1 = [0]

The maximum value of constant can be 255.

Read and Write should be considered legacy instruction. Should not be used in lab 2 onwards - though compiler should still be able to compile them.

Control operations: No control operation supported in today’s lab.

Language support labels. Rules:

- Labels have to start with . character

- They should be the first character of any line (no space or tab should prefix them).

- Label will alphanumeric string of characters. No spacial symbol, no :

- Line having label will have no other expression

Example:

.label1

.loop

Control operations:

B{Suffix}  <label>

Suffix given in table below:

Example:

BEQ .loop

BGE .label1

Table is taken from [REF1]

More information about these condition flags  [ARM]

Data movement operation:

<Dest Variable> = <constant value>

Variable names: x0 to x255.

Constant value: 0 to 255

Support for comments. Any character following % will be ignored by parser.

Example:  Sum of an array. Assume size of Array is stored at address 0, elements stored at 0x4 onwards location. Final result will be stored at address = size * 4 + 4.

x1 = 0       % index

x2 = 0        % sum

x3 = 0        % array address

x4 =  [x3]    %  read the size of array

.loopback

x15 = x1 - x4   % compare

BEQ .exit

x3 = x3 + 4    % increase the address

x6 =  [x3]       % load the array data

x2 = x2 + x6  % update sum

x1 = x1 + 1    % increment index

BAL .loopback

.exit

[x3] = x2

Example bytecode output of the above program is (written in hex)

F 1 0 0

F 2 0 0

F 3 0 0

5 4 0 3

2  F 1 4

10 0 0 6

9 3 3 4

5 6 0 3

1 2 2 6

9 1 1 1

1E 0 0 FA

6 3 0 2

# Byte code specification

Compiler generates a byte code for every operation and finally the byte code is stored in file called “program.byte”.

Format of each line of byte code

<opcode>  <dest register> <operand 1> <operand 2>

For branch instructions:

<opcode>  <0> <0>  <relative offset of jump location>

Relative offset could be positive or negative, such that:

next instruction address = current instruction address + offset.

| Operation | Opcode if second operand is a variable | Opcode if second operand is a constant |
| --- | --- | --- |
| Add | 0x01 | 0x09 |
| Subtract | 0x02 | 0x0A |
| Multiply | 0x03 | 0x0B |
| Divide | 0x04 | 0x0C |
| Memory read | 0x05 | 0x0C |
| Memory write | 0x06 | 0x0E |
| Data movement | 0x07 | 0x0F |
| Branch | 0x10 + code given in branch table |  |

In case of memory read, write and data movement operations - operand 1 is 0.

Note that

- Every line has four bytes

- The maximum value for each byte is 255.

# Processor specification

Processor

Instruction memory

Size 256 byte

Data memory

Size 256 byte

Processor has four functions:

reset()

fetch()

decode()

execute()

Following data elements are there in processor:

Int Register[256]

Int PC, opcode, dest, src1, src2

extern char Instruction[256], Data[4096];

Int end_of_simulation = 0

It has a global array of registers of size 256. And a global variable named PC. Also global flags N Z C V

Reset()  initializes each element of register array to 0 as well initilize PC to 0

fetch(): reads four bytes from the PC address of instruction memory and stores in opcode, dest, src1, and src2 respectively

decode() : void and empty function at this time

execute(): based on opecode, it performs the operations. Once see 0 opcode, set end_of_simulation = 1

# Memory

Memory is coded as a separate ‘c’ file.

Char Instruction[256];

Char Data[4096];

Assumes program.byte and data.byte present in the directory. The memory is byte addressable which means when you read instructions from char array will read the instruction bye-by-byte.

data.byte and program.byte are stored as an array of hex bytes. In each line four bytes in hex format (without prefix 0x) separated by space are stored

For example -1 is represented as

FF FF FF FF

initialize() {

//read the data from “program.byte” and populate Instruction memory

//read the data from “data.byte” and populate data memory.

}

finalize() {

//Write data.byte

}

# Main file

main(int argc, char** argv) {

compile();

initialise();

reset();

while(end_of_simulation) {

fetch();

decode();

execute();

}

finalize();

}

It is suggested that the program file is passed as argument to your final executable - using argc and argv.

# Directory structure

Following code files should be there:

main.c

compiler.c

compiler.h

processor.c

processor.h

memory.c

memory.h

Makefile

Some tutorials on multi-file compilation:

https://www.codewithc.in/tutorials/multi-file-programs

https://www.w3schools.com/c/c_organize_code.php

You can search more on the internet

Makefile tutorial:

https://makefiletutorial.com/

# FAQs

Example user level programs:

- Sum of N numbers where N is fixed at program time

- Multiply two complex numbers

- Find the determinant of a 3x3 matrix.

Following can be done when branch instructions are introduced.

- Sum of N numbers where N is given at run time.

  - Four bytes starting from 0x0 stores N

  - Next 4N bytes store the N numbers.

- FIR filter response:

  - Input start at 0x0 of data memory. First four bytes tells size of array, say N.  Next 4N byes are input array.

  - Output is stored at 0x10.

  - Weights are part of the program - as constants.

Data size

- In this assignment, data size is not specified. You can assume each operation is 8 bits, or 32 bits. 32 bit operation size is preferable. Data size is always 32 bit.

Submission Instructions:

- Submit as a single zipped file - containing all your code, and test programs.